Program Manager Implementation Document 

Programs are fundamentally an extension of the existing LV Calculator. They depend on the LV calculator. 

At its’ simplest, the Program mode is in service of one thing: Making missions out of rockets.

The missions it simulates could be as simple as delivering a sat to GTO, and as complex as an Artemis Program moon landing. It could even be more complex. It could be like Constellation’s missions to mars, with docking, refueling, orbital habs, oh my!

Bare minimum, the program manager needs to offer two things:

- Determine  the leftovers a stage has once it’s reached orbit (remaining prop determined by the LV calculator)

- Allow a user to define what to do with the stage after. Burn, separate, etc, all while correctly penalizing the fuel of a stage after.

On a more complex level, the program manager should enable the user to:

- Calculate how many stages of a vehicle are needed to make a user-defined target orbit,

- Accurately report what the fuel load of the stage would be after meeting the delta-v requirements to get to orbit

- Carry multiple payloads to orbit

- Perform maneuvers in-orbit

- Separate those payloads

- Independently use the payloads as controllable vehicles to do their own maneuvers

- Launch multiple more rockets into the same orbit, each upper being controllable, and

- Interact with the other vehicles on-orbit in various, useful ways.

Examples of useful interactions include:

- Rendezvous

- Docking

- Propellant transfer

- Crew Transfer

- Maneuvering

- Separating

- Expending (removes a stage from active vehicles after a given point.)

- Reentering

- Recovered

New Categories:

- Spacecraft

  - A Spacecraft is an array of stages, with specific, crew-based, propellant transfer-based, metadata.

    - Example: An Apollo LM Spacecraft would be defined by a descent stage, with no crew capabilities, docking capabilities, or propellant transfer capabilities, and an ascent stage, with crew capacity up to two, docking abilities, and no propellant transfer capabilities.

- Spaceports

  - A Spaceport is an already-defined object, which will be extended to contain an array of pads with location metadata that determines orbits.

- Pads

  - Pads are a specific launch complex within a spaceport that allow for launches.

- Launch Vehicles:

  - LV’s already exist and are central to the LV calculator. The Program mode extends them in a non-destructive way: adding new Payload sections, so the Launch Vehicle can take Spacecraft to orbit. They can take multiple payloads.

  - LV upper stages must be turned into spacecraft. Using calculations to find the amount of leftover fuel in an upper stage after reaching orbit with payload, the upper stage must be defined as a spacecraft, with user-editable fields, same as other spacecraft.

How should this all come together? What’s the ideal workflow?

- The user should start off loading a pre-defined rocket (with minimal editing capability) and using a new “spacecraft editor” (heavily based off the vehicle editor section of the LV calculator) to load them as “payload” onto the rocket they selected.

  - The spacecraft editor should be able to

    - Make new spacecraft

    - Have pre-loaded spacecraft

    - Load spacecraft from JSON files the user has saved

    - Create multi-stage spacecraft with the user being able to configure all stages and new spacecraft metadata, i.e. docking, propellant transfer, crew capability, etc.

    - Provide the user with useful feedback, such as a delta-v breakdown per-stage, based off the one in the Vehicle Calculator.

- The user should then be able to interact with the actual mission manager itself. The mission manager will include:

  - A right-hand side panel, which allows users to add new events to the mission (LAUNCH, RENDEZVOUS, DOCKING, PROP TRANSFER, EXPENDING, SEPARATING, MANEUVER), with events being associated with specific vehicles at certain points in time

    - Events should be removable and movable in order. Clicking on an event should bring up more of it’s details.

  - A large central panel, featuring two modes, which should be swappable via a toggle in the upper-right hand corner:

    - Vehicle state vs. time. Time should not be defined as a literal, linear scale of time, as this would lead to extremely bunched-up events happening in short succession, but with event-based separations.

      - Vehicles on the central panel should be represented by lines, roughly corresponding to their orbital state. This can be roughly represented by height.

      - When multiple vehicles are in play, they are represented by different lines, separated, and color-coded. The user should be able to modify the color coding.

      - Events should be marked as circles on the vehicle’s line.

      - As long as a stage is not RECOVERED or EXPENDED, it should have a “plus” sign in a circle at the end of it’s line, enabling the user to add more events there.

      - A large, vertical line which moves when the user clicks on the panel, which is used by other vehicles to determine the state at which to reflect a vehicle. Essentially, it determines at what point in the mission other panels can reflect the vehicles state at.

    - Orbit management panel. This should represent different orbits as nodes on a map, with switching between planets and SOIs done intentionally. This is fundamentally the home of the MANEUVER.

      - Automatic calculations of delta-V when a user makes a bridge between two nodes.

      - Automatic insertion of a MANEUVER into EVENTS after creating it.

      - A large, left-hand panel, which takes orbits from the ORBITS page of the LV calculator and allows the user to make new nodes via it.

  - A lower-left side live propellant and delta-V monitoring system box, using the vertical line to determine what state it should be measuring the vehicle in.

Critical Assumptions (Grandfathered from previous program document):

### Physics Model

Patched conics, impulsive burns, Hohmann/bi-elliptic transfers, SOI transitions

modeled as instantaneous. Ascent losses use the existing Townsend-Schilling model

from the LV Calculator output.

**Rule 2 — No stage is automatically expended.**

When a stage separates or exhausts its propellant, it becomes an independent

FlightVehicle with whatever propellant remains. The user decides what happens to

it next. Nothing is expended without a deliberate user action.

*Implication: an S-IVB with 200 kg residual prop is still a live vehicle that

can perform small burns, dock with a depot, or be explicitly expended.*

**Rule 3 — Docking is free once orbits match.**

The last significant maneuver is matching orbital state (altitude + inclination + LAN).

Once two vehicles share the same OrbitalState, docking is treated as instantaneous

with zero ΔV cost. No proximity ΔV. No rendezvous phasing. Same orbit → can dock.

**Rule 5 — ΔV is only consumed by engine burns.**

BURN events consume ΔV and propellant. All other events (SEPARATE, DOCK,

TRANSFER_CREW, TRANSFER_PROPELLANT, LAND) consume neither.

When a SEPARATE event fires at position N of the stack (Should be labeled clearly, using a named stage, so the USER knows what’s going on):

FlightVehicle A: stages[0..N-1]  (lower portion)

FlightVehicle B: stages[N..]     (upper portion)

Both get new vehicleIds and their own Band View lanes.

### 3.8 OrbitalState

```

{

body:        string,   // "Earth" | "Moon" | "Mars" | "Venus"

apogee:      km,

perigee:     km,

inclination: deg,

lan:         deg,

epoch:       T+_seconds,

surface:     boolean

}

```

Most math is already outlined in program_math_reference.md. USE IT.